Y3

Yamaha R6 2003-2005

| Kit for left side: | Kit for right side: |
| --- | --- |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel adapter | 578 | 1 |
| 2 | Poly.cylind. diameter =16mm | X10 | 1 |
| 3 | Poly.cylind.diameter= 23,5mm | X15 | 1 |
| 4 | Bolt M10X60 | 850 | 1 |
| 5 | Dural redu.cylinder | 1 | 1 |
| 6 | Steel box | X1 | 1 |
| 7 | Dural.cylind. L=5 | 16 | 1 |
| 8 | Bolt M10x45x1,25 | 926 | 1 |
| 9 | 3M Tape | 3M | 1 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel adapter | 577 | 1 |
| 2 | Poly.cylind. diameter =16mm | X10 | 1 |
| 3 | Poly.cylind.diameter= 23,5mm | X15 | 1 |
| 4 | Bolt M10X60 | 850 | 1 |
| 5 | Dural redu.cylinder | 1 | 1 |
| 6 | Steel box | X1 | 1 |
| 7 | Dural.cylind. L=17 | 158 | 1 |
| 8 | Bolt M10x55x1,25 | 915 | 1 |
| 9 | 3M Tape | 3M | 1 |

Instruction for left side:

- Take off fairing.

- Take off original bolt M10 from frame.

- Wher is adapter touching with frame, stick 3M stickers ID.9.

- ID.2 screwing on  thread on the steel adapter ID.1.

- Insert steel box ID.6 into the protector, insert bolt ID.4 and under the protector put ID.5. This kit mount into the adapter, from back side screwing polyamid cylinder ID.3.

- This fitting kit mount on the frame by bolt  ID.8, into the frame insert dural.cylinder ID.7.

- Put fairnig back on the bike.

Instruction for right side:

- Take off fairing.

- Take off original bolt M10 from frame.

- Wher is adapter touching with frame, stick 3M stickers ID.9.

- ID.2 screwing on  thread on the steel adapter ID.1.

- Insert steel box ID.6 into the protector, insert bolt ID.4 and under the protector put ID.5. This kit mount into the adapter, from back side screwing polyamid cylinder ID.3.

- This fitting kit mount on the frame by bolt  ID.8, into the frame insert dural.cylinder ID.7.

- Put fairnig back on the bike.

Polyamid nut, which is nut for cash bobbins should be resting on the frame, so that when the fall down don´t occurred spin kit.

We recomanded use the glue on threads. Medium strong.