Y45SD

Yamaha MT-09 2017-

Mounting Kit – Left Side:		     		Mounting Kit – Right Side:

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. adapter SD | 537 | 1 |
| 2 | Dural. sleeve l=75 | 1236 | 1 |
| 3 | Bolt M10x130x1,25 | 905 | 1 |
| 4 | Bolt M8x25 | 815 | 2 |
| 5 | Washer M8 | 800 | 2 |
| 6 | Spring washer M10 | 801 | 1 |

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. adapter SD | 537 | 1 |
| 2 | Dural. sleeve l=53 | 1235 | 1 |
| 3 | Bolt M10x110x1,25 | 928 | 1 |
| 4 | Bolt M8x25 | 815 | 2 |
| 5 | Washer M8 | 800 | 2 |
| 6 | Spring washer M10 | 801 | 1 |

Mounting - Left Side:

- Remove original bolts from the frame.

- Thread the M10 washer ID.6 and dural. sleeve ID.2 on the bolt ID.3.  Sleeve ID.2 by mounting in the frame.

- Install this assembly into the motorcycle frame.

- Mount the slider using M8 bolts ID.4 and washers ID.5

Mounting - Right Side:

- Remove original bolts from the frame.

- Thread the M10 washer ID.6 and dural. sleeve ID.2 on the bolt ID.3.  Sleeve ID.2 by mounting in the frame.

- Install this assembly into the motorcycle frame.

- Mount the slider using M8 bolts ID.4 and washers ID.5.

Warning! We recommend using medium strength thread glue (blue) during assembly.

1

2

3

4

5

6