Y45S

Yamaha MT-09 2017-

Mounting kit - Left Side:		     		Mounting Kit – Right Side:

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Dural.SL adapter d=73 | 360 | 1 |
| 2 | Bolt M10x105x1,25 | 893 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Dural.SL adapter d=63 | 337 | 1 |
| 2 | Bolt M10x95x1,25 | 927 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |

Mounting Instructions Left Side:

- Unscrew original bolt from frame.

- Equip ID.2 bolt with ID.1 dural. adapter so that it faces the frame.

- Mount this assembly onto the frame.

- Finally, use ID.3 bolts to fix the slider and ID.4 bolts to fix the dural. plate.

Mounting Instructions Right Side:

- Unscrew original bolt from frame.

- Equip ID.2 bolt with ID.1 dural. adapter so that it faces the frame.

- Mount this assembly onto the frame.

- Finally, use ID.3 bolts to fix the slider and ID.4 bolts to fix the dural. plate.

Note: We recommend using thread glue during assembly.

1

2

3

4