Y45

Yamaha MT-09 2017-

Fittng kit left side:		     		  Fitting kit right side:

| ID | Description | Cat. | QTY |
| --- | --- | --- | --- |
| 1 | Steel box | X1 | 1 |
| 2 | Al.spacer d=40mm | 1237 | 1 |
| 3 | Bolt M10x115x1,25 | 894 | 1 |

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 1 | Steel box | X1 | 1 |
| 2 | Alu.spacer d=28mm | 1238 | 1 |
| 3 | Bolt M10x100x1,25 | 892 | 1 |

Instruction to assembly left side:

- Take out original bolt from frame.

- Insert steel box ID.1 into the protektor.

- Put bolt ID.3 into the protector, unrer the protector put dural.spacer ID.2 and into the frame insert ID.2.

- This set mount into the frame.

Instruction to assembly right side:

- Take out original bolt from frame.

- Insert steel box ID.1 into the protektor

- Put bolt ID3 into the protector and put on it al.cylinder ID.2.

- This set mount into the frame.

Warning: Use the glue of all threads. Medium strong.