Y4

Yamaha FZS 600 Fazer 98-03

Fitting kit left side:				Fitting kit right side:

| ID. | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 581 | 1 |
| 2 | Spacer d=10mm | 108 | 1 |
| 3 | Bolt M8x110 | 879 | 1 |
| 4 | Bolt M10x110 | 844 | 1 |
| 5 | Bolt cone M12x40 | 858 | 1 |
| 6 | Alu spacer under protector | 237 | 1 |
| 7 | Steel box | X2 | 1 |
| 8 | Bolt M12x40 | 857 | 1 |
| 9 | Nut M10 | 810 | 1 |
| 10 | Nut M8 | 809 | 1 |
| 11 | Washer M10 | 804 | 1 |
| 12 | Washer M8 | 802 | 1 |
| 13 | Plastic cover | X4 | 1 |
| 14 | Plastic cover | X5 | 1 |

| ID. | Description | Cat. | Qt |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 580 | 1 |
| 2 | Spacer d=10mm | 108 | 1 |
| 3 | Bolt M8x110 | 879 | 1 |
| 4 | Bolt M10x110 | 844 | 1 |
| 5 | Bolt cone M12x40 | 858 | 1 |
| 6 | Alu spacer under protector | 283 | 1 |
| 7 | Steel box | X2 | 1 |
| 8 | Bolt M12x40 | 857 | 1 |
| 9 | Nut M10 | 810 | 1 |
| 10 | Nut M8 | 809 | 1 |
| 11 | Washer M10 | 804 | 1 |
| 12 | Washer M8 | 802 | 1 |
| 13 | Plastic cover | X4 | 1 |
| 14 | Plastic cover | X5 | 1 |

4

8

9

10

12

3

7

5

1

6

4

3

2

10

11

7

6

8

1

9

5

2

Instruction assembly:

- Assembly left and right side adapter as shown on the picture.

- Dismantle original bolts from frame. Done left side firstly.

- Assebmly ID.6 by bolts ID.5 and tighten it early than you put aluminum adapter on the bike.

- Don´t forget place alu. spacer ID.2 betewwn the frame and adapter.

- If you have mounted alu. adapter ID.1 with spacer ID.6 on the motorcycle, assembly protectors.

- Insert ID.7 into the protectors and use bolts ID.8 for tightening on the adapter.

Use medium strong glue of all threads during assembly.

3

2

4

1