Y8SD

Yamaha YZF-R1

2002-2003

| Left side kit |  |  |  |  | Right side kit |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  | ID | Description | Cat.no. | Pcs |  |
| 1 | SLD adapter | 537 | 1 |  | 1 | SLD adapter | 537 | 1 |  |
| 2 | Bolt M10x90x1,25 | 890 | 1 |  | 2 | Bolt M10x90x1,25 | 890 | 1 |  |
| 3 | Washer M10 | 801 | 1 |  | 3 | Washer M10 | 801 | 1 |  |
| 4 | Dural. cylinder 20mm | 173 | 1 |  | 4 | Dural. cylinder 13mm | 172 | 1 |  |
| 5 | Bolt M8x25 | 815 | 2 |  | 5 | Bolt M8x25 | 815 | 2 |  |
| 6 | Washer M8 | 800 | 2 |  | 6 | Washer M8 | 800 | 2 |  |
| 7 | Cylinder adapter | 10 | 1 |  | 7 | Cylinder adapter | 10 | 1 |  |

Instrucions

- Remove original bolts from frame of motorcycle according to the picture below.

- Prepare bolt ID2 with washer ID3 and put it through the SLD adapter ID1, cylinder adapter ID7 and cylinder ID4. Mount this kit into the frame of motorcycle.

- Fit slider using two bolts ID5 and washers ID6.

We recommend to use the glue on all threads (medium strong).