Y9-2

Yamaha R1 2004-2006

Kit for left side:		     		            Kit for right side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x120x1,25 | 895 | 1 |
| 3 | Dural.cylind. d=62,5 | 498 | 1 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x120x1,25 | 895 | 1 |
| 3 | Dural.cylinder d=55 | 407 | 1 |

3

1

3

2

2

1

Instruction to assembly:

- The first do assembly left side.

- Take the protector for left side, insert steel box ID.1 into the protector.

- Insert bolt through the protector and after insert dural.cylinder ID.3 on the bolt.

- This kit gripping on the frame, replace original bolt M10.

- Mounting instruction for right side is same so left side.

Warning: We recommend use the glue of all threads. Medium strong.

2

1

2

3

1

3