Y9

Yamaha R1 2004-2006

| Kit for left side: | Kit for right side: |
| --- | --- |

| ID. | Description | Cat.N | Qt |  | ID. | Description | Cat.N | Qt |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Steel box | X1 | 1 |  | 7 | Steel box | X1 | 1 |
| 8 | Bolt M10x60 | 850 | 1 |  | 8 | Bolt M10x60 | 850 | 1 |
| 9 | Alu.spacer d=20mm | DV2 | 1 |  | 9 | Alu.spacer d=10mm | DV1 | 1 |
| 10 | Bolt M10x70x1,25mm | 898 | 1 |  | 10 | Bolt M10x100x1,25mm | 892 | 1 |
| 11 | Alu. spacer d=16,5mm | 106 | 1 |  | 11 | Alu. spacer d=27,5mm | 107 | 1 |
| 12 | Dural. protector holder | 583 | 1 |  | 12 | Dural. protector holder | 582 | 1 |
| 14 | Plastic cover | X4 | 1 |  | 14 | Plastic cover | X4 | 1 |
| 15 | 3M tape |  | 1 |  | 15 | 3M tape |  | 1 |
| 17 | Washer M10 | 804 | 1 |  | 17 | Washer M10 | 804 | 1 |

7

8

7

9

8

9

11

10

10

11

12

12

17

17

14

14

15

Assembly- left side:

- Remove the original bolt M10.

- Bolt ID.10 insert into the dural. adapter ID.12 with washer ID.17, place alu. spacer ID.11, stick 3M tape on the frame, where is adapter touching with chassis.

- Place plastic cover ID.14.

- Place steel box ID.7 into the protector. Insert bolt ID.8.

- Under the protector place alu. spacer ID.9.

Assembly- right side:

- Remove the original bolt M10.

- Bolt ID.10 insert into the dural. adapter ID.12 with washer ID.17, place alu. spacer ID.11, stick 3M tape on the frame, where is adapter touching with chassis.

- Place plastic cover ID.14.

- Place steel box ID.7 into the protector. Insert bolt ID.8

- Under the protector place alu. spacer ID.9.

Warning- we recommand to use glue of threads during assembly. Medium strong.

(ilustr. photo, place for fitting kit)